Olusegun Afolabi

EECE 412

Prof Wenjie

Introduction

The objective of Lab 12 was to extend the existing Pong Game Starter by implementing a firing mechanism. The player now controls a spaceship (modified paddle) that can launch a missile (ball) horizontally across the screen. This lab introduces animation, bitmap rendering, conditional launching, and projectile motion using a firing button.

Steps and Implementation

1. Project Setup

- A new Vivado project was created based on the existing Pong framework.

- The source file lab12_Afolabi_pong_graph_st.vhd was created and added to the project.

- The .xdc constraints file was updated to include BTN4 for firing control, along with existing paddle movement buttons.

2. Shape Definition and Display

- A spaceship was defined using a 16x16 ROM bit-map.

- Drawing logic compares the current pixel_x and pixel_y values to the bounding box of the spaceship.

- A ROM lookup determines whether a pixel should be drawn.

- The spaceship is rendered using a blue color (graph_rgb <= "001").

3. Missile Launch Logic

- A missile (firing ball) object was introduced with a 4x4 bounding box.

- A missile launch occurs only when:

  - BTN4 is pressed.

  - The previous missile has exited the screen (single shot).

- The missile spawns at the center of the spaceship and moves horizontally left across the screen on each refresh cycle.

4. Object Movement

- Movement control uses refr_tick, which triggers at each vertical screen refresh (60 Hz).

- Each refresh decrements the missile’s x position by a constant speed (BALL_V := 4).

- If the missile is not fired or moves off-screen, it remains hidden.

Code:

Lab Results

3a.

The firing ball was defined using a custom 8x8 bitmap ROM

4a.

Launch control checks the fire button and resets missile firing when the ball moves off-screen

4b.

When the missile is not fired, it tracks the spaceship center. When fired, it continuously moves left at a fixed speed.

4c. Bounding logic for the missile is handled as:

ROM mapping to draw the pixel:

4d.

When the missile pixel is active (rd_ball_on = '1'), it is drawn on top. Otherwise, the spaceship or background is shown depending on pixel conditions.

Conclusion

Lab 12 successfully introduced a firing ball projectile system into the Pong-based VGA game framework. Through bitmap shape definitions, button-triggered movement, and screen-based object tracking, the lab demonstrated advanced control of graphics at the pixel level. These skills are foundational for future projects involving real-time object dynamics, embedded game development, and FPGA-based graphics systems.